TECHNOLOGICAL UNIVERSITY OF THE PHILIPPINES

Ayala Boulevard, Ermita, Manila

CIT-ELECTRONICS DEPARTMENT

CPET11L-M – Microprocessor and Microcontroller Systems, Lab

1st Semester, SY 2-24-2025

| Name: Joseph C. Arenas | Instructor: Prof. Tristan Mopas |
| --- | --- |
| Course & Section: BET-CPET- 3A | Date Submitted: |

Activity 2

Topic 1: Arduino 7-Segment Display Counter Push Button

Topic 2: Segment Display Count Up Arduino

Topic 3: Arduino Push Button 2-Digit 7-Segment

Topic 4: Arduino Countdown Timer in Minutes & Seconds

- OBJECTIVES

- To apply practical knowledge in using the Arduino Uno R3 and Mega

- To explain the functions and components of the related topics

- To implement the 7-Segment Display, Dual Digit 7-Segment Display, and 4-Digit 7-Segment Display using code and circuit diagrams

- To develop and enhance problem-solving skills related to the topics

- EQUIPMENT AND MATERIALS

HARDWARE

- Arduino Uno or Arduino Mega 2560

- Breadboard

- Jumper Wires

- Push Button

- Laptop

- 7-Segment Display & 4-Digit 7-Segment Display

- Resistors 220Ω and 10KΩ

- Buzzer

SOFTWARE

- Arduino IDE

- MS Word

- TinkerCad Simulator

- Wokwi Simulator

- DIAGRAM

===== Topic 1: Arduino 7-Segment Display Counter Push Button =====

- TinkerCad Simulation

- Breadboard

if(digit !=2)

digitalWrite(c,HIGH);

if(digit != 1 && digit !=4 && digit !=7)

digitalWrite(d,HIGH);

if(digit == 2 || digit ==6 || digit == 8 || digit==0)

digitalWrite(e,HIGH);

if(digit != 1 && digit !=2 && digit!=3 && digit !=7)

digitalWrite(f,HIGH);

if (digit!=0 && digit!=1 && digit !=7)

digitalWrite(g,HIGH);

}

void turnOff(){

digitalWrite(a,LOW);

digitalWrite(b,LOW);

digitalWrite(c,LOW);

digitalWrite(d,LOW);

digitalWrite(e,LOW);

digitalWrite(f,LOW);

digitalWrite(g,LOW);

}

Source Code

const int a = 8;

const int b = 9;

const int c = 4;

const int d = 5;

const int e = 6;

const int f = 2;

const int g = 3;

bool bPress = false;

const int buttonPin = 10;

int buttonPushCounter = 0;

int buttonState = 0;

int lastButtonState = 0;

void setup() {

pinMode(a, OUTPUT);  //A

pinMode(b, OUTPUT);  //B

pinMode(c, OUTPUT);  //C

pinMode(d, OUTPUT);  //D

pinMode(e, OUTPUT);  //E

pinMode(f, OUTPUT);  //F

pinMode(g, OUTPUT);  //G

pinMode( buttonPin , INPUT_PULLUP );

Serial.begin(9600);

displayDigit(buttonPushCounter);

}

void displayDigit(int digit){

if(digit!=1 && digit != 4)

digitalWrite(a,HIGH);

if(digit != 5 && digit != 6)

digitalWrite(b,HIGH);

===== Topic 2: Segment Display Count Up Arduino =====

- TinkerCad Simulation

- Breadboard

byte pin[] = {2, 3, 4, 5, 6, 7, 8, 9};

int number[9][8] = {

{1, 1, 0, 0, 0, 1, 1, 1},

{0, 0, 1, 0, 0, 0, 1, 0},

{1, 0, 0, 0, 0, 0, 1, 0},

{1, 1, 0, 0, 0, 1, 0, 0},

{1, 0, 0, 0, 1, 0, 0, 0},

{0, 0, 0, 0, 1, 0, 0, 0},

{1, 1, 0, 0, 0, 0, 0, 1},

{0, 0, 0, 0, 0, 0, 0, 0},

{1, 1, 0, 0, 0, 0, 0, 0},

};

Source Code

void setup() {

for (byte a = 0; a < 8; a++) {

pinMode(pin[a], OUTPUT);

}

}

void loop() {

for (int a = 0; a < 9; a++) {

for (int b = 0; b < 8; b++) {

digitalWrite(pin[b], number[a][b]);

}

delay(500);

}

}

=== TOPIC 3: LED with combination of Potentiometer and Switch ===

- Wokwi Simulation

- Breadboard

- Source Code

int leds_Uno[] = {2, 3, 4, 5, 6, 7, 8};

int leds_Dos[] = {22, 23, 24, 25, 26, 27, 28};

int button_Uno = 9;

int button_Dos = 10;

int numbers = 0;

int digit_Array[10][7] = {

{1, 1, 1, 1, 1, 1, 0},

{0, 1, 1, 0, 0, 0, 0},

{1, 1, 0, 1, 1, 0, 1},

{1, 1, 1, 1, 0, 0, 1},

{0, 1, 1, 0, 0, 1, 1},

{1, 0, 1, 1, 0, 1, 1},

{1, 0, 1, 1, 1, 1, 1},

{1, 1, 1, 0, 0, 0, 0},

{1, 1, 1, 1, 1, 1, 1},

{1, 1, 1, 1, 0, 1, 1},

};

void setup(){

for(int i = 0 ; i < 7 ; i++){

pinMode(leds_Uno[i], OUTPUT);

pinMode(leds_Dos[i], OUTPUT);

}

pinMode(button_Uno, INPUT);

pinMode(button_Dos, INPUT);

}

void segmentOutput(int pins[], int digit){

for(int i = 0 ; i < 7 ; i++){

digitalWrite(pins[i], digit_Array[digit][i]);

}

}

void loop(){

if (digitalRead(button_Uno) == HIGH){

numbers--;

if (numbers < 0) numbers = 99;

delay(150);

}

if (digitalRead(button_Dos) == HIGH){

numbers++;

if (numbers > 99) numbers = 0;

delay(150);

}

int tens = numbers / 10;

int ones = numbers % 10;

segmentOutput(leds_Uno, tens);

segmentOutput(leds_Dos, ones);

}

=== Topic 4: Arduino Countdown Timer in Minutes & Seconds ===

- Wokwi Simulation

- Breadboard

- Source Code

#include <math.h>

int digit_pin[] = {6, 9, 10, 11}; // PWM Display  digit pins from left to right

int speakerPin = 15;

#define DIGIT_ON  LOW

#define DIGIT_OFF  HIGH

int segA = 2;

int segB = 3;

int  segC = 4;

int segD = 5;

int segE = A0; //pin 6 is used bij display 1 for  its pwm function

int segF = 7;

int segG = 8;

//int segPD = ;

int  button1=13;

int button2=12;

int button3=16;

int button4=17;

int  countdown_time = 60;

struct struct_digits {

int digit[4];

};

void  setup() {

pinMode(segA, OUTPUT);

pinMode(segB, OUTPUT);

pinMode(segC, OUTPUT);

pinMode(segD, OUTPUT);

pinMode(segE, OUTPUT);

pinMode(segF, OUTPUT);

pinMode(segG, OUTPUT);

for (int i=0; i<4;  i++) {

pinMode(digit_pin[i], OUTPUT);

}

pinMode(speakerPin,  OUTPUT);

pinMode(button1,INPUT_PULLUP);

pinMode(button2,INPUT_PULLUP);

pinMode(button3,INPUT_PULLUP);

pinMode(button4,INPUT_PULLUP);

}

void  playTone(int tone, int duration) {

for (long k = 0; k < duration * 1000L; k  += tone * 2) {

digitalWrite(speakerPin, HIGH);

delayMicroseconds(tone);

digitalWrite(speakerPin, LOW);

delayMicroseconds(tone);

}

}

case  3:

digitalWrite(segA, SEGMENT_ON);

digitalWrite(segB, SEGMENT_ON);

digitalWrite(segC, SEGMENT_ON);

digitalWrite(segD, SEGMENT_ON);

digitalWrite(segE,  SEGMENT_OFF);

digitalWrite(segF, SEGMENT_OFF);

digitalWrite(segG,  SEGMENT_ON);

break;

case 4:

digitalWrite(segA, SEGMENT_OFF);

digitalWrite(segB, SEGMENT_ON);

digitalWrite(segC, SEGMENT_ON);

digitalWrite(segD,  SEGMENT_OFF);

digitalWrite(segE, SEGMENT_OFF);

digitalWrite(segF,  SEGMENT_ON);

digitalWrite(segG, SEGMENT_ON);

break;

case  5:

digitalWrite(segA, SEGMENT_ON);

digitalWrite(segB, SEGMENT_OFF);

digitalWrite(segC, SEGMENT_ON);

digitalWrite(segD, SEGMENT_ON);

digitalWrite(segE,  SEGMENT_OFF);

digitalWrite(segF, SEGMENT_ON);

digitalWrite(segG, SEGMENT_ON);

break;

void  lightNumber(int numberToDisplay) {

#define SEGMENT_ON  HIGH

#define SEGMENT_OFF  LOW

switch (numberToDisplay){

case 0:

digitalWrite(segA,  SEGMENT_ON);

digitalWrite(segB, SEGMENT_ON);

digitalWrite(segC, SEGMENT_ON);

digitalWrite(segD, SEGMENT_ON);

digitalWrite(segE, SEGMENT_ON);

digitalWrite(segF,  SEGMENT_ON);

digitalWrite(segG, SEGMENT_OFF);

break;

case  1:

digitalWrite(segA, SEGMENT_OFF);

digitalWrite(segB, SEGMENT_ON);

digitalWrite(segC, SEGMENT_ON);

digitalWrite(segD, SEGMENT_OFF);

digitalWrite(segE, SEGMENT_OFF);

digitalWrite(segF, SEGMENT_OFF);

digitalWrite(segG, SEGMENT_OFF);

break;

case 2:

digitalWrite(segA,  SEGMENT_ON);

digitalWrite(segB, SEGMENT_ON);

digitalWrite(segC, SEGMENT_OFF);

digitalWrite(segD, SEGMENT_ON);

digitalWrite(segE, SEGMENT_ON);

digitalWrite(segF,  SEGMENT_OFF);

digitalWrite(segG, SEGMENT_ON);

break;

case 9:

digitalWrite(segA, SEGMENT_ON);

digitalWrite(segB,  SEGMENT_ON);

digitalWrite(segC, SEGMENT_ON);

digitalWrite(segD, SEGMENT_ON);

digitalWrite(segE, SEGMENT_OFF);

digitalWrite(segF, SEGMENT_ON);

digitalWrite(segG, SEGMENT_ON);

break;

case 10:

digitalWrite(segA,  SEGMENT_OFF);

digitalWrite(segB, SEGMENT_OFF);

digitalWrite(segC,  SEGMENT_OFF);

digitalWrite(segD, SEGMENT_OFF);

digitalWrite(segE,  SEGMENT_OFF);

digitalWrite(segF, SEGMENT_OFF);

digitalWrite(segG,  SEGMENT_OFF);

break;

}

}

void SwitchDigit(int  digit) {

for (int i=0; i<4; i++) {

if (i == digit) {

digitalWrite(digit_pin[i],  DIGIT_ON);

} else {

digitalWrite(digit_pin[i], DIGIT_OFF);

}

}

}

case 6:

digitalWrite(segA, SEGMENT_ON);

digitalWrite(segB,  SEGMENT_OFF);

digitalWrite(segC, SEGMENT_ON);

digitalWrite(segD, SEGMENT_ON);

digitalWrite(segE, SEGMENT_ON);

digitalWrite(segF, SEGMENT_ON);

digitalWrite(segG,  SEGMENT_ON);

break;

case 7:

digitalWrite(segA, SEGMENT_ON);

digitalWrite(segB, SEGMENT_ON);

digitalWrite(segC, SEGMENT_ON);

digitalWrite(segD,  SEGMENT_OFF);

digitalWrite(segE, SEGMENT_OFF);

digitalWrite(segF,  SEGMENT_OFF);

digitalWrite(segG, SEGMENT_OFF);

break;

case  8:

digitalWrite(segA, SEGMENT_ON);

digitalWrite(segB, SEGMENT_ON);

digitalWrite(segC, SEGMENT_ON);

digitalWrite(segD, SEGMENT_ON);

digitalWrite(segE,  SEGMENT_ON);

digitalWrite(segF, SEGMENT_ON);

digitalWrite(segG, SEGMENT_ON);

break;

bool Countdown(int n, int del){

int minutes = n / 100;

int seconds = n % 100;

while (minutes > 0 || seconds > 0) {

// Combine back into MMSS format for display

int displayTime = minutes * 100 + seconds;

PrintNumber(displayTime, del);

if (digitalRead(button2) == LOW) {

return false; // stop pressed

}

// countdown logic like a real clock

if (seconds == 0) {

if (minutes > 0) {

minutes--;

seconds = 59;

}

} else {

seconds--;

}

}

// Final 00:00

PrintNumber(0,0);

playTone(1519,1000);

return true;

}

struct struct_digits IntToDigits(int n){

struct struct_digits  dig;

int zeros=0;

int d;

for (int i=0; i<4; i++) {

d=n/pow(10,3-i);

zeros += d;

n = n - d*pow(10,3-i);

if (zeros!=0 || i==3) {

dig.digit[i]=d;

} else {

dig.digit[i]=10;

}

}

return dig;

}

void PrintNumber(int n, int time) {

struct struct_digits  dig;

dig = IntToDigits(n);

for (int i=0; i<= time/20; i++) {

if (digitalRead(button2)==LOW) {

return;

}

for (int j=0;  j<4; j++) {

SwitchDigit(j);

lightNumber(dig.digit[j]);

delay(5);

}

}

}

else if (digitalRead(button4)==LOW) {

if (pressed4 == 0 || pressed4 > 30) {

int minutes = countdown_time / 100;

int seconds = countdown_time % 100;

// cap at 99:59 to stay within 4 digits

if (!(minutes == 99 && seconds == 59)) {

if (seconds == 59) {

if (minutes < 99) { minutes++; seconds = 0; }

} else {

seconds++;

}

countdown_time = minutes * 100 + seconds;

dig = IntToDigits(countdown_time);

}

}

pressed4 += 1;

}

if (digitalRead(button3)==HIGH)  pressed3 = 0;

if (digitalRead(button4)==HIGH)  pressed4 = 0;

}

}

void reset() {

int m, zeros, d, pressed3  = 0, pressed4 = 0;

countdown_time = 1200;

struct struct_digits dig;

dig = IntToDigits(countdown_time);

while (digitalRead(button1)==HIGH)  {

for (int j=0; j<4; j++) {

SwitchDigit(j);

lightNumber(dig.digit[j]);

delay(5);

}

// Decrement (one second, borrow to minutes when needed)

if (digitalRead(button3)==LOW) {

if (pressed3 == 0 || pressed3 > 30) {

int minutes = countdown_time / 100;

int seconds = countdown_time % 100;

if (minutes > 0 || seconds > 0) {

if (seconds == 0) {

if (minutes > 0) { minutes--; seconds = 59; }

} else {

seconds--;

}

countdown_time = minutes * 100 + seconds;

dig = IntToDigits(countdown_time);

}

}

pressed3 += 1;

}

void loop(){

reset();

while (!Countdown(countdown_time,962))  {

reset();

}

while (digitalRead(button2)==1){};

}

- PROCEDURE

A. Preparation

- Gather the required components: Arduino Uno R3, breadboard, LEDs, resistors, potentiometer, switches, and jumper wires.

- Install the Arduino IDE on the computer.

- Review the circuit diagrams for each topic.

B. Actual

- Connect the LED with a potentiometer according to the diagram and upload the code.

- Set up the LED with a switch and test the on/off function.

- Combine the potentiometer and switch to control the LED in different ways.

C. Checking

- Observe if the LED responds correctly to the potentiometer and/or switch.

- Verify the wiring connections and the uploaded code.

- Adjust values or fix errors if the output is not working as expected.

D. Uploading

- Upload the final and corrected code to the Arduino Uno R3.

- Ensure that all three topics (potentiometer, switch, and combination) work properly.

- CONCLUSION

The activities highlighted the practical integration of hardware and software using Arduino, particularly with 7-segment displays and LEDs. They demonstrated how digital logic, coding, and circuit design work together to produce functional outputs. Overall, the tasks strengthened both technical understanding and problem-solving skills in microcontroller applications.